| Pattern | Sample Image | Average Picture Level (APL) | Usage | Advantages | Disadvantages |
| --- | --- | --- | --- | --- | --- |
| 3-Bar Video Signal |  | 50% | - Has been the default luminance test pattern for the display industry - Initially developed for use with CRT displays | - Already referenced by ENERGY STAR, CEC and IEC - Pattern widely available on DVD or Blu-ray test discs - Common sense appeal – midway between full white and full black | - APL higher than typical TV program content - No reference point for vertical centering - Forces plasma displays into a power limiting state, producing a lower luminance level than with test patterns with lower APLs - Industry experts recommend moving to a pattern more representative of broadcast content - IEC is considering moving to a new test pattern in the next revision of 62087 - Static pattern – not representative of dynamic video typical of TV program content |
| 9-Point Video Signal |  | 17% | - Used by Chinese TV test procedure and ISO/IEC test procedure for front projectors | - Arguably, achieves more accurate absolute measurements for plasmas - White target is in the middle of screen | - As part of a luminance ratio may disproportionately disadvantage some plasmas - APL lower than typical TV program content - Static pattern – not representative of dynamic video typical of TV program content |
| DOE 5-Point Video Signal |  | 34% | - Developed by DOE as part of this proposed test procedure | - APL of typical broadcast content - White target is in the middle of screen - Similar in appearance to an available VESA 5-point pattern (RT01BN) | - Static pattern – not representative of dynamic video typical of TV program content |
| Dynamic Video Signal |  | ~34% | - In development by DOE and IEC as part of this proposed test procedure and IEC’s next update | - APL of typical broadcast content - Not a static image – representative of dynamic video typical of TV program content - May be taken forward by IEC and become part of IEC 62087 update. | - Pattern not yet finalized by DOE and IEC - Measurement procedure yet to be determined |

This pattern was created using the IEC 62087 2008 test clip with black and white squares in the center of the screen as measurement points.